- B-52 18” SUB SPECS AND BASS PRO CABINET SUGGESTIONS:

- With a low moving mass, 3.5" voice coil, and powerful ferrite motor B-52 designed the 18-190S to deliver high output bass with incredible impact. Originally used in the MX-18S passive subwoofer, the 18-190S has incredible output capability when used in compact vented enclosures and has plenty of excursion to produce gut wrenching low frequency response. In addition, with a rigid cast aluminum frame and rugged construction the 18-190S is built to withstand the use and abuse of the professional sound environment.

- Product Specifications

- Nominal Diameter 18"

- Power Handling (RMS) 550 Watts

- Power Handling (max) 1100 Watts

- Impedance 8 ohms

- Frequency Response 30 to 640 Hz

- Sensitivity 96.67 dB 2.83V/1m

- Magnet Weight 190 oz.

- Thiele-Small Parameters

- Resonant Frequency (Fs) 22.8 Hz

- DC Resistance (Re) 5.89 ohms

- Voice Coil Inductance (Le) 2.27 mH

- Mechanical Q (Qms) 4.94

- Electromagnetic Q (Qes) 0.24

- Total Q (Qts) 0.23

- Compliance Equivalent Volume (Vas) 15.37 ft.³

- Mechanical Compliance of Suspension (Cms) 0.24 mm/N

- BL Product (BL) 26.57 Tm

- Diaphragm Mass Inc. Airload (Mms) 200.9g

- Maximum Linear Excursion (Xmax) 6 mm

- Surface Area of Cone (Sd) 1128 cm²

- Materials of Construction

- Cone Material Paper

- Surround Material Cloth

- Cast Aluminum Frame

- Voice Coil Wire Material Copper

- Voice Coil e® / Polyimide

- Basket / Frame Material Cast Aluminum

- Magnet Material Ferrite

- Mounting Information

- Overall Outside Diameter 18.5"

- Baffle Cutout Diameter16.875"

- Depth 7"

- Bolt Circle Diameter 17.5"

- Number Of Mounting Holes 8

- Optimum Cabinet Size - Determined using BassBox 6 Pro High Fidelity Suggestion

- Sealed Volume1.69 ft.³

- Sealed F3 81 Hz

- Vented Volume 2.4 ft.³

- Vented F3 57.2 Hz

- THIS WOOFER WAS USED IN THE B-52 Subwoofer: MX-18S 18" 550W Subwoofer.

- Frequency response of any MX series full-range system to its lowest octave with the B-52 MX-18S 18" 550W Subwoofer. The MX-18S uses an American-made (Celesrion) cast-frame 18" subwoofer with a 3.5" high-temperature polyamide voice coil. The rugged speaker cabinet features metal bar handles, 16-gauge perforated steel grille, rubber feet, and protective metal corners. Finished with impact-resistant black carpet. Input connectors include 2 - 1/4" metal phone jacks and a genuine Neutrik NL4 connector.

- FEATURES

- 300W

- High-performance loudspeaker and 1" exit titanium diaphragm compression driver

- Metal bar handles

- 16-gauge perforated steel grille

- Rubber feet

- Protective metal corners

- 2 - 1/4" metal phone jack inputs

- Neutrik NL4 input

- Impact-resistant black carpet

- Pole mount adaptor

SPECIFICATIONS:

- This sub uses 4 ports of unspecified diameter and unspecified length

- Nominal impedance: 8 ohms

- Frequency response: 30-160Hz

- Power handling: 550W

- SPL @ 1w1m: 99dB

- Peak SPL: 130dB

- Dimensions: 23"W x 26"H x 22"D

- Weight: 90 lbs.